Power System Analysis

EE2600

Exam 4

Date: 02-05-2022

Answer all the questions. If required, make suitable assumptions and state those.

Time: 3 hours                                                                                                                      Full marks: 30

Q1.                                                                                                                                                                     6

Consider a 3-phase transposed overhead transmission line. Each phase of the transmission line comprises a single solid conductor. All the conductors have the same radius and are located at the same height from the ground. The information about the radius and height of a conductor is provided in the excel sheet. The center-to-center distance between two consecutive conductors is also mentioned in the excel sheet. If the length of the transmission line is 500 km, find out the phase-to-phase and phase-to-ground capacitance values of the transmission line for its entire length. Please consider the effect of ground on line capacitances.

Q2.                                                                                                                                                                      6

Consider a 500 km long, 3-phase, distributed parameter, balanced transmission line. The information about the line parameters with reference to its positive sequence network is provided in the excel sheet. The system frequency is 50 Hz. The line is terminated with a load of power factor 0.8 (leading). The receiving end voltage and current magnitudes (for the positive sequence network of the transmission line) are given in the excel sheet. Find out the 3-phase active power and reactive power fed to the transmission line at the sending end. The overall system is assumed to be balanced.

Q3.                                                                                                                                                                     6

The information of the cost function coefficients and power generation limits of four generators is provided in the excel sheet. The network loss is assumed to be 3% of the total power produced by the first three generators. Find out the optimal generator outputs and the minimum generation cost to serve a total load of 3000 MW.

Q4.                                                                                                                                                                    6

Consider a simple two-bus power system. The transmission line connecting the buses is of the medium length. The sending end bus voltage magnitude is 1 p.u. The information about the active and reactive power loads at the receiving end is provided in the excel sheet. The receiving end bus is essentially a  bus. The line parameter data is also provided in the excel sheet. Find out the active and reactive power losses in the transmission line as well as the receiving end bus voltage magnitude via the Gauss-Seidel iterative technique. Do not use any acceleration or deceleration factor. Produce the final result after five iterations. Ignore the shunt conductance. As usual, the initial values of all the unknown bus voltage magnitudes and angles should be taken as 1 p.u. and 0 radian, respectively.

Q5.                                                                                                                                                                  6

Consider the following simultaneous non-linear algebraic equations. The values of , , , ,  and  are provided in the excel sheet. You need to solve the given equations by means of Newton-Raphson iterations without using any acceleration or deceleration factor. Start with the initial solution as per the excel sheet data. Produce the updated solutions from the first and second iterations. Please do not worry about the convergence of the result.

-End-