Power System Analysis

EE2600

Exam 4

Date: 01-05-2024

Answer all the questions. If required, make suitable assumptions and state those. All the angles are expressed in radian in this question paper. Please try to maintain computational accuracy upto four decimal places.

Time: 3 hours                                                                                                                      Full marks: 30

Q1.                                                                                                                                                                     5

The conductor arrangement of a 3-phase transposed transmission line over a particular transposition section is shown below. Here, each phase comprises two conductors, and the radius of each conductor is 2 cm. Find out the inductance per unit length of the transmission line for its positive sequence equivalent circuit over its entire length.

Q2.                                                                                                                                                                     5

Consider the positive-sequence equivalent circuit of a 3-phase lossless distributed-parameter transmission line. The electrical length of the transmission line is defined as the product of its length and the phase constant, and is symbolized as θ. The sending end voltage angle is δ radian and the receiving end voltage angle is 0 radian. If both the sending end and receiving end voltage magnitudes are Vt  Volt, prove that the mid-point voltage (indicated by Vm) of the transmission line would be given by the following expression. Assume -π2≤δ≤π2.

Vm=Vtcosδ2cosθ2

Q3.                                                                                                                                                                     5

The receiving end active power and reactive power of a single-phase distributed-parameter transmission are 102.73 MW and -10.173 MVAR, respectively, when the line is terminated with its characteristic impedance. The sending end and receiving end voltage phasors of the transmission line are observed to be  212∠0.5760 kV and 200 kV, respectively, when the line is terminated with its characteristic impedance. Find out the voltage and current phasors at the mid-point of the transmission line.

Q4.                                                                                                                                                                     5

Refer to the optimization problem shown below. It is guessed that only the first and last inequality constraints may become active at the optimal solution point. Find out the solution for primal and dual variables on the basis of the above assumption. You also need to verify whether the solution thus obtained is really the optimal solution or not. The whole analysis should be carried out on the basis of the KKT necessary conditions of optimality.

Minimize  x2+2y2+z2-5x-10y-z

s.t.

x-2y+z≤-3-2x-y+4z≥-95x-2y+6z≤7

Q5.                                                                                                                                                                     5

Use the Newton-Raphson method to solve the following simultaneous non-linear algebraic equations. Report the results of first two iterations. The initial values of all the variables may be assumed to be zero.  No acceleration factor should be used. Do not forget to show the expression of the Jacobian matrix. Note that the problem that you are solving here is basically a distributed-slack load flow problem. Here, Pslack is expressed in per unit.

10cosδ1+π2+4cosδ2+π2-0.5Pslack=3

10cos-δ1+π2+5cosδ2-δ1+π2-0.3Pslack=-2

4cos-δ2+π2+5cosδ1-δ2+π2-0.2Pslack=-1.5

Q6.                                                                                                                                                                    5

In a four-bus network, Bus 1 is connected to Bus 2, Bus 3, and Bus 4 via transmission lines of series impedances (with reference to the equivalent π-model) equal to j0.1 p.u.,  j0.2 p.u.,  and j0.25 p.u.,  respectively.  The shunt admittances may be ignored. In the case the bus voltages are 1∠0 p.u.,  0.95∠-π/15 p.u, 1.01∠π/15 p.u, and 0.98∠-π/12 p.u, respectively, find out the active and reactive power injections at Bus 1 by following the bus admittance matrix approach.

-End-